General Safety Information

WARNING	– To avoid serious injuries:

SPD pedals are designed so that you release them only when you

intend to release. They are not designed to release automatically

at times such as when falling off the bicycle.

This is a safety measure to prevent you from losing balance and

falling off the bicycle as a result of your feet becoming

accidentally released from the pedals.

Use only SPD shoes with this product. Other types of shoes may

not release from the pedals, or may release unexpectedly.

Use only Shimano cleats (SM-SH51/ SM-SH56) and tighten the

mounting bolts securely to the shoes.

Before attempting to ride with these pedals and shoes, make sure

you understand the operation of the engagement / release

mechanism for the pedals and cleats (shoes).

Cleat types and using the pedals

WARNING

Do not use the pedals and cleats in any way other than as

described in these Service Instructions. The cleats are

designed to engage and release from the pedals when the

cleats and pedals are facing forward.

See below for instructions on how to install the cleats.

FAILURE TO FOLLOW THESE INSTRUCTIONS MAY

RESULT IN SERIOUS PERSONAL INJURY.

Single release mode cleats (SM-SH51) and multiple

release mode cleats (SM-SH56) are available for use with

these pedals.

Each type of cleat has its own distinctive features, so be

Releasing the cleats from the pedals

The method of release varies according to the type of cleats you are using.

(Check the model number and color of your cleats to determine the proper method of release.)

Single release mode cleats:	Multiple release mode cleats:

SM-SH51 (black)	SM-SH56 (gold/Optional accessory)

These cleats only release when the heel is twisted outward.	The cleats can be released by twisting in any direction.

They will not release if the heel is twisted in any other direction.

You are able to apply upward force to the pedal, since they will not	Because the cleats can also be released by lifting your heel, they may

release unless the foot is twisted outward.	become accidentally released when applying upward force on the

pedals.

These cleats should not be used for riding style which may involve

pulling up on the pedals or for activities, such as jumping, in which

pulling forces are applied to the pedals causing possible release from

the pedals.

The cleats will not necessarily release if you lose your balance.	Although the cleats can be released by twisting your heels in any

Accordingly, for places and conditions where it looks as though you may	direction or by lifting, they will not necessarily release if you lose your

Pay attention to

the mark

R: right pedal

L: left pedal

15 mm spanner

PD-M520

Tightening torque:

35 N·m {304 in. lbs.} min.

Before you attempt to ride with these pedals and shoes, apply the

brakes, then place one foot on the ground and practice engaging

and releasing each shoe from its pedal until you can do so

naturally and with minimal effort.

Ride on level ground first until you become adept at engaging and

releasing your shoes from the pedals.

Before riding, adjust the spring tension of the pedals to your

liking.

When riding at low speed or when there is a possibility that you

might need to stop riding, (for example, when doing a U-turn,

nearing an intersection, riding uphill or turning a blind curve),

release your shoes from the pedal beforehand so that you can

quickly put your feet onto the ground at any time.

Use a lighter spring tension for attaching the pedal cleats when

riding in adverse conditions.

Keep cleats and bindings clear of dirt and debris to ensure

engagement and release.

If you do not maintain both your shoes and cleats in good

condition, release and engagement to the pedals could become

unpredictable or difficult, which could result in severe injury.

Remember to check the cleats periodically for wear. When the

cleats are worn, replace them.

Always check the spring tension after replacing the pedal cleats

and before riding.

Be sure to attach reflectors to the bicycle when riding at night.

Do not continue riding the bicycle if the reflectors are dirty or

damaged, otherwise it becomes more difficult for oncoming

vehicles to see you.

Obtain and read the service instructions carefully prior to

installing the parts. Loose, worn, or damaged parts may cause

serious injury to the rider.

We strongly recommend only using genuine Shimano

replacement parts.

If you have any questions concerning your pedals, contact a

sure to read the Service Instructions carefully and select

whichever type of cleat best suits your riding style and

the terrain and riding conditions which you will

encounter.

Other types of cleats such as single release mode cleats

(SM-SH52) and multiple release mode cleats (SM-SH55)

cannot be used, as they do not provide stable enough

step-out performance or sufficient holding force.

Engaging the cleats with the pedals

Press the cleats into the pedals with a forward and downward motion.

SI-41R0F

PD-M970

PD-M540	SPD Pedals

PD-M520

lose balance, make sure that you have sufficient time to release the	balance.

cleats beforehand.	Accordingly, for places and conditions where it looks as though you may

lose balance, make sure that you have sufficient time to release the

cleats beforehand.

When you are pedaling vigorously, your heel may inadvertently twist	If the cleats release by accident, you may fall off the bicycle and serious

outward and this may cause the cleat to release accidentally. If the cleat	injury may result.

releases by accident, you may fall off the bicycle and serious injury may	If the features of multiple release mode cleats are not sufficiently

result.	understood and if they are not used correctly, then they may release

You can help to prevent this by adjusting the amount of force required to	accidentally with a greater frequency than for single release mode

release the cleats, and you should practice until you are accustomed to	cleats.

the amount of force and the angle required to release the cleats.	You can help to prevent this by adjusting the amount of force required to

release the cleats in any direction, and you should practice until you are

accustomed to the amount of force and the angle required to release.

Single release mode	Multiple release mode

Release the cleats from the pedals by twisting	Release the cleats from the pedals by twisting

your heels outward.	your heels in any direction.

Note:	Note:

It is necessary to practice releasing	In multiple release mode, it is

until you become accustomed to	necessary to practice releasing

the technique.	until you become accustomed to

the technique. Releasing by lifting

your heel requires particular

practice.

| 3. | From the bottom of the shoe, position a cleat and then a cleat adapter over the cleat holes. The cleats are compatible with both left and rightWaterproof seal |
| --- | --- |

pedals.  Provisionally tighten the cleat mounting bolts.

Remove the sockliner and attach the waterproof seal.

Provisional tightening torque

for cleat mounting bolts:	Note:	Waterproof seal

2.5 N·m {22 in. lbs.}	The waterproof seal is

supplied with Shimano shoes

which require this step to be

4 mm Allen key	carried out.

Adjusting the spring tension of the binding

The spring force is adjusted by means of adjustment bolts. The adjustment

bolts are located behind each of the bindings, and there are two

adjustment bolts on each pedal. Equalize the tensions by referring to the

adjustment plates and by counting the number of turns of the adjustment

bolts. The spring tension can be adjusted in four steps for each turn of the

adjustment bolt.

Adjustment bolt

Adjustment plate

2

1

Adjustment bolt

Decrease	Increase

2	1

PD-M970: 2.5 mm Allen key

PD-M540 / PD-M520: 3 mm Allen key

When the adjustment bolt is turned clockwise, the spring tension

increases, and when it is turned counterclockwise, the spring tension

decreases.

Strongest position	Weakest position

Adjustment

bolt

professional dealer.

Read these Technical Service Instructions carefully, and keep

them in a safe place for later reference.

BE SURE TO READ AND FOLLOW THE ABOVE WARNINGS

CAREFULLY.

If the warnings are not followed, your shoes may not come

out of the pedals when you intend or they may come out

unexpectedly or accidentally, and severe injury may result.

Technical Service Instructions

Attaching the cleats

| 1. | With a pair of pliers or a similar tool, pull off the rubber cover to expose the cleat mounting holes. |
| --- | --- |

Note:

This step may not be necessary depending

Front

Cleat mounting bolts

Cleat adapter

Cleat

Sockliner

Adjustment plate

If the adjustment plate is at the strongest or the weakest position,

do not turn the adjustment bolt any further.

Note:

In order to prevent accidental release from occurring, make sure

all the spring tensions are properly adjusted.

NOTE:

Check that there is no looseness in any joints or connections

before riding the bicycle. (BB-FC, FC-PD)

In addition, if pedaling performance does not feel normal, check

this once more.

An optional reflector set (SM-PD22) is available. Please contact a

professional dealer for details.

Parts are not guaranteed against natural wear or deterioration

resulting from normal use.

For maximum performance we highly recommend Shimano

on the type of shoes.

Rubber cover for cleat

mounting holes

SPD shoe

2. Remove the sockliner and position a cleat nut over the oval holes.

Cleat nut

Note:

Position the triangular portion

of the cleat toward the front

of the shoe.

Adjusting the cleat position

1. The cleat has an adjustment range of 20 mm front to

back and 5 mm right to left. After provisionally tightening

the cleat, practice engaging and releasing, one shoe at

a time. Readjust to determine the best cleat position.

2. After you have determined the best cleat position,

Mounting the pedals on the crank arms

Use an 8 mm Allen key (for PD-M970 / PD-M540 pedals) or a 15 mm

spanner (for PD-M520 pedals) to mount the pedals on the crank arms.

The right pedal has a right-hand thread; the left pedal has a left-hand

thread.

PD-M970

PD-M540

The spring tensions should be adjusted equally on both sides of

each pedal and for both right and left pedals. If they are not

adjusted equally, it can cause the rider difficulty to engage or

release from the pedals.

Cleat replacement

Cleats wear out over time and should be replaced periodically. Cleats

should be replaced when it becomes difficult to release, or it starts to

release with much less effort than the when it was in new condition.

Maintenance of the axle units

lubricants and maintenance products.

This step may not be necessary

depending on the type of shoes.

Sockliner

firmly tighten the cleat mounting bolts with a 4 mm

Allen key.

Tightening torque:

5 – 6 N·m {43 – 52 in. lbs.}

8 mm Allen key

Tightening torque:

35 N·m {304 in. lbs.} min.

If you experience any trouble with the rotating parts of the pedal, the pedal

may require adjustment. Obtain advice from a professional dealer.

One Holland, Irvine, California 92618, U.S.A.  Phone: +1-949-951-5003	Industrieweg 24, 8071 CT Nunspeet, The Netherlands  Phone: +31-341-272222

3-77 Oimatsu-cho, Sakai-ku, Sakai, Osaka 590-8577, Japan

Please note: Specifications are subject to change for improvement without notice. (English)

C  Mar. 2006 by Shimano Inc. PIT. IZM. Printed in Malaysia

SM-SH51

R

PD-M520 MALAYSIA VIA-M R

15